Model W5944M-HT RFID Tag		Specifications & Data Sheet

The Model W5944M-HT high temperature Tag is capable of surviving the unprecedented temperature of 300°C (572°F), providing identification and tracking capabilities never-before available in rugged or hazardous use-areas.  This Tag can be mounted to any metallic surface by bolting, riveting, or welding and endure high pressure environmental conditions.

| Model Number | Part Number | Description | Read Range (on metal) |
| --- | --- | --- | --- |
| W5944M-HT | 807-0016-W5944M-HT | Model W5944M-HT Weldable Metal Mount High Temp 300°C Tag | A99H Antenna – 9’ A55C Antenna – 7’ A64C Antenna – 6’ |

| Mechanical: | Measurements: | 59x44x15mm | 2.3”x1.7”x.59” |
| --- | --- | --- | --- |
|  | Material: | Stainless Steel Shell | Ceramic |
|  | Weight: | 39 grams | 1.4 Ounces |
| Certifications: | RoHS III | REACH | CE |
|  | FCC Part 15 | ETSI | Free of BBP, DEHP, DBP, DIBP |

| Alien Higgs 3: | TID – 64 bits | UID/EPC – 96-480 bits | 512 bits |
| --- | --- | --- | --- |
| RF: | Standard: | EPC Class1 Gen2 | ISO 18000-6C |
|  | Frequency: | 902 – 928 MHz (North America) | 860 – 870 MHz (EU, Japan) |
|  | Power: | Passive, no Battery | Powered by Reader RF Signal |
| Chip Options (page 2): | TID – 32 to 208 bits | EPC – 0 to 496 bits | User – 0 to 61,400 bits |

| Environmental: | Temp, Peak:. Temp, Operating:. Life:. Ingress Protection:. | +350°C/+700°F @ 1 hour duration. -85°F to +185°F. 40 Year Shelf Life. IP68. | -30°C to +85°C. 100k Writes. |
| --- | --- | --- | --- |

Available UHF Memory Chips:

| Alien Higgs3 Higgs4 HiggsEC Higgs9 | TID (ROM). 64 bits. 64 bits. 48 bits. 48 bits. | UID (EPC). 96 to 480 bits. 128 bits. 96-128 bits. 496 bits. | User Memory. 512 bits. 128 bits. 128 bits. 688 bits. |
| --- | --- | --- | --- |
| EM EM4123 EM4124 EM4126 EM4324 EM4325 EM4423 dual NFC/UHF | TID (ROM). 64 bits. 64 bits. 32 bits. 64 bits. 48 bits. 96 bits. | UID (EPC). 0 bits. 96 bits. 208 bits. 96 bits. 352 bits. 64-160 bits. | User Memory. 0 bits. 0 bits. 0 bits. 720 bits. 3072 bits. |
| Fujitsu (FRAM) MB97R88110 MB97R88120/8130 MB97R8050 | TID (ROM). 208 bits. 208 bits. 176 bits. | UID (EPC). 480 bits. 480 bits. 160 bits. | User Memory. 61,400 bits. 61,400 bits. 0 bits. |
| Impinj M4D M4E M4QT M4i MX-8k Monza R6P Monza R6 Monza R6A Monza R6B Monza 5 M730 M750 | TID (ROM). 96 bits. 96 bits. 96 bits. 96 bits. 96 bits. 96 bits. 96 bits. 96 bits. 96 bits. 96 bits. 96 bits. 96 bits. | UID (EPC). Up to 128 bits. Up to 496 bits. Up to 128 bits. Up to 256 bits. Up to 128 bits. Up to 128 bits. 96 bits. 96 bits. Up to 128 bits. 128 bits. 128 bits. 96 bits. | User Memory. 32 bits. 128 bits. 512 bits. 480 bits. 8,192 bits. Up to 64 bits. 0 bits. 0 bits. 32 bits. 32 bits. 0 bits. 32 bits. |
| NXP UCODE 7 UCODE 7m UCODE 7xm UCODE 7xm+ UCODE 8 UCODE 8m UCODE 9 UCODE G2iL & G2iL+ UCODE G2iM UCODE G2iM+ UCODE G2XM UCODE G2XL UCODE HSL | TID (ROM). 48 bits. 48 bits. 48 bits. 48 bits. 96 bits. 96 bits. 96 bits. 64 bits. 96 bits. 96 bits. 64 bits. 64 bits. 64 bits. | UID (EPC). 128 bits. 128 bits. 448 bits. 448 bits. 128 bits. 96 bits. 96 bits. 128 bits. 256 bits. Up to 448 bits. 240 bits. 240 bits. 0 bits. | User Memory. 0 bits. 32 bits. 1024 bits. 2048 bits. 0 bits. 32 bits. 0 bits. 0 bits. 512 bits. Up to 640 bits. 512 bits. 0 bits. 1680 bits. |